| Company logo | Manufacturing SOP | Doc. No. | SOP1 |
| --- | --- | --- | --- |
|  | SOP NAME | Date | 10/10/2025 |
|  |  | Revision No. | 01 |

| Purpose: To outline the procedure for safely and correctly installing an air conditioner in a caravan. |  |  |
| --- | --- | --- |
| Procedure: - Step 1: Inspect the roof for correct installation procedure and clean the surface using a rag. - Step 2: Mark the mounting points on the roof for the brackets and drill holes for the bracket. Make sure not to damage any wires or insulation. - Step 3: Secure the mounting brackets using Tek screws and bolts using a torque wrench to 10Nm (Recommended Spec). - Step 4: Lift and place the air con unit on the roof and secure it to the brackets using the provided bolts. - Step 5: Apply SMP 50 silicone sealant around the base of the air con to prevent any leakage. - Step 6: Wipe out any extra silicone residue and check for the seals to ensure they are tight and smooth - Step 7: Connect the air conditioner wires to the electrical systems and test the connection. - Step 8: Turn on the air conditioner to check if it is working properly. |  | Photos: |
| Reference Document (if any): - Air conditioner installation manual |  |  |
| Roles and Responsibilities: |  |  |
| Role: Production Operator (Roof section) | Responsibilities: Ensure the assembly steps are followed precisely, inspect parts for defects, and document any discrepancies. |  |
| Required Tools and Equipment: - Power tools - Sealants (SMP 50) - Mounting brackets - Ladder - Silicon gun - Torque wrench | PPE: - Safety glass - Ear plug - Safety gloves - Steel caped boot |  |